DLI Edge AI and Robotics Teaching Kit

Module 4 Lab: 
Autonomous Jetbot Navigation with Notebook

If you have a JetBot, the best way to get started with autonomous navigation is to use the starter Jupyter Notebooks at the NVIDIA-AI-IOT/Jetbot project.

Using Pre-built SD Image:

You can use an SD image that contains all the required software including the JetBot project and Jupyter Notebooks. The SD image differs from the SD images used in Module 1 as it uses a slightly older version of the L4t (currently 4.5) and contains additional libraries and software to enable the notebooks to automatically load when you start the JetBot. Go to https://jetbot.org/master/software_setup/sd_card.html  and download the image for either 2 GB Jetson Nano or the 4 GB Jetson Nano depending on what version you have.

*Note - Many third party JetBot kits such as the Sparkfun Jetbot AI Kit may already provide a pre-flashed SD card and is configured with drivers that support the hardware accessories in their kits.

Follow the instructions at https://jetbot.org/master/software_setup/sd_card.html from Step 1 to Step 5 to set the SD image, configure wi-fi and connect remotely to the Jupyter-lab server using a web browser.

Alternative using existing SD Image and Docker Container:

If you already have an SD image you wish to work with, you can follow the instructions at https://jetbot.org/master/software_setup/docker.html.

Follow steps 1 through step 5 to enable wi-fi, clone the JetBot project from Github and enable the container. Note that this version has been targeted for Jetpack 4.4.

Log into web browser to remotely access your JetBot:

There are five folders.  My example in the image shows six but the collision_road is a community project that might not exist in your your JetBot folder.

- Basic Motion – covers the basics of controlling your JetBot

- Collision Avoidance – next you will learn to train your JetBot to avoid obstacles. You will start with the Data Collection notebook and then train a classification model on the data you collected to build a model and test it out on your JetBot.*

- Object Following – The Object Following section contains a live demo of using a pretrained neural network on the Coco dataset that identifies 90 different objects and then direct your JetBot to follow an identified object.

- Road Following – In this section, you will again collect data, but you will be using a regression technique rather than classification to collect and label the path that your JetBot should follow. Then you will train the collected dataset and test it out on your JetBot.*

- Teleoperation – This section contains a notebook that shows you how to test and operate a connected gamepad as a alternative to using your keyboard for controlling your Jetbot.

* For the Collision Avoidance and Road Following sections, you can opt to download the collected data and training a different GPU machine that may have more memory and processing power and then copy the generated model to your JetBot.