DLI Edge AI and Robotics Teaching Kit

Module 6 Lab: 
Natural Language Conversational AI – Text to Speech

This lab assigns sections from the jetson-voice repository on Github. For more information, the content for the entire repository can be found in the readme documents at: https://github.com/dusty-nv/jetson-voice.

This lab is meant to run on any of the Jetson Developer kits.  The document contains all the instructions to run the lab based on the jetson-voice repository.

For the System Setup, we will be using a Docker Container.

Initial System Setup using Docker:

You will first clone the development branch of the jetson-reinforcement repository on Github. Execute the following at the ubuntu terminal:

git clone --branch dev https://github.com/dusty-nv/jetson-voice.git

Navigate into the directory you just cloned:

cd jetson-voice

Run the container

Jetson – l4t:

Run a shell script. You will run this command each time you start up your device and need to run the container.

docker/run.sh

Change and update the code

In your terminal, install the PyDictionary module.

pip install PyDictionary

Navigate into the directory which you will find tts.py

cd jetson-voice/examples

Update the code in the current tts.py file with the tts_solution.py

Run Sample - Text to speech sample

Navigate to jetson-voice directory

cd ~/Desktop

To test the ASR sample on a mic, first list the audio devices in your system to get the audio device ID's:

scripts/list_audio_devices.sh

The examples/tts.py app can output the audio to a speaker, wav file, or sequence of wav files. The following code will take the input, find out the synonyms and will play the audio over the speaker.

examples/tts.py --output-device <> --output-wav data/audio/tts_test

It may take some time to run the sample application.